XDF

1.110000

DO NOT HAND EDIT!!!! (Trust me)

%%HEADER%%

001000 FileVers         ="0.9"

001005 DefTitle         ="89 165 ECM 6E And Stuff"

001007 DescSize         =0x29

001006 Desc             ="This is for 1227165 $6E (1989 F-Body and "

001010 Author           ="Mark Mansur"

001030 BinSize          =0x4000

001035 BaseOffset       =0

001200 ADSAssoc         ="..\\..\\..\\TunerPro RT\\DataStreams\\1227165_6E.ads"

001225 ADSCheck         =0x0

001300 GenFlags         =0x0

001325 ModeFlags        =0x0

002000 Category0        ="Fuel"

002001 Category1        ="Spark"

002002 Category2        ="IAC"

002003 Category3        ="Fan"

002004 Category4        ="Idle"

002005 Category5        ="BLM"

002006 Category6        ="INT"

%%END%%

%%FLAG%%

000002 UniqueID         =0x93

030005 Title            ="Manual Transmission"

030011 DescSize         =0x14

030010 Desc             ="0 = MANUAL XMISSION"

030100 Address          =0x14

030200 BitNumber        =0x0

%%END%%

%%FLAG%%

000002 UniqueID         =0x92

030005 Title            ="Single Fire Mode"

030011 DescSize         =0x15

030010 Desc             ="1 = SINGLE FIRE MODE"

030100 Address          =0x14

030200 BitNumber        =0x1

%%END%%

%%FLAG%%

000002 UniqueID         =0x91

030005 Title            ="Analog MAF Meter in use (HLM)"

030011 DescSize         =0x23

030010 Desc             ="2 = ANALOG MAF METER IN USE, (HLM)"

030100 Address          =0x14

030200 BitNumber        =0x2

%%END%%

%%FLAG%%

000002 UniqueID         =0x8F

030005 Title            ="VATS Enable"

030011 DescSize         =0x12

030010 Desc             ="4 = VATS ENABLE\r\n"

030100 Address          =0x14

030200 BitNumber        =0x4

%%END%%

%%FLAG%%

000002 UniqueID         =0x8E

030005 Title            ="Closed Loop for Can Purge"

030011 DescSize         =0x36

030010 Desc             ="5 = Req Closed Loop For Canister Purge\r\n             "

030100 Address          =0x14

030200 BitNumber        =0x5

%%END%%

%%FLAG%%

000002 UniqueID         =0x8D

030005 Title            ="Use TCC for Shift Lamp Control"

030011 DescSize         =0x3A

030010 Desc             ="6 = USE TCC FOR SHFT LAMP CNT'L\r\n                        "

030100 Address          =0x14

030200 BitNumber        =0x6

%%END%%

%%CHECKSUM%%

000002 UniqueID         =0x1

010005 Title            ="Imported Checksum"

010010 DataStart        =0x8

010015 DataEnd          =0x3FFF

010025 StoreAddr        =0x6

010030 CalcMethod       =0x0

010050 Flags            =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2

020005 Title            ="Prom ID"

020011 DescSize         =0x42

020010 Desc             ="This is a chip's ID. Use this for keeping track of chip versions."

020020 Units            =" "

020050 SizeInBits       =0x10

020100 Address          =0x0

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4D6

020005 Title            ="Date Code Year"

020011 DescSize         =0xA

020010 Desc             ="Born Date"

020020 Units            =" "

020050 SizeInBits       =0x10

020100 Address          =0x2

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3

020005 Title            ="Sequence Number Month Day"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            =""

020050 SizeInBits       =0x10

020100 Address          =0x4

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4

020005 Title            ="Checksum"

020011 DescSize         =0xD0

020010 Desc             ="Checksum is used to verify the contents of a bin/chip. If the checksum does not match the contents, the car may not run correctly (Limp Home Mode), unless the checksum is disabled (0xAA in the mask field).  "

020020 Units            =" "

020050 SizeInBits       =0x10

020100 Address          =0x6

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4536

020005 Title            ="Mask ID (Set to AA to disable Checksum)"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            =" "

020100 Address          =0x8

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5

020005 Title            ="Number of Cylinders $00=8 $C0=6 $80=4"

020011 DescSize         =0x38

020010 Desc             ="0x00 = 8 Cylinder\r\n0xC0 = 6 Cylinder\r\n0x80 = 4 Cylinder"

020020 Units            =" Cylinders"

020100 Address          =0x9

020200 Equation         =X+8,TH|0|0|0|0|

020205 OutType          =0x2

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6

000100 Cat0ID           =0x2

020005 Title            ="Spark Reference Angle - Base Timing"

020011 DescSize         =0x34

020010 Desc             ="This should be set to match the distributor timing."

020020 Units            ="Degrees"

020100 Address          =0x1C

020200 Equation         =0.351567 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1D

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7

000100 Cat0ID           =0x2

020005 Title            ="Spark Advance Maximum "

020011 DescSize         =0x5A

020010 Desc             ="This is the maximum advance the computer will incur.              (Relative to Reference)"

020020 Units            ="Degrees"

020100 Address          =0x1E

020200 Equation         =0.351567 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1EA9

000100 Cat0ID           =0x2

020005 Title            ="Spark Retard Maximum "

020011 DescSize         =0x5D

020010 Desc             ="This is the maximum retard the computer will incur.                  (Relative to Reference)"

020020 Units            ="Degrees"

020100 Address          =0x21

020200 Equation         =0.351567 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x397A

000100 Cat0ID           =0x2

020005 Title            ="Spark Advance Fixed For Diagnostic"

020011 DescSize         =0x19

020010 Desc             ="Fixed Spark Adv For Diag"

020020 Units            ="Degrees"

020100 Address          =0x22

020200 Equation         =0.351567 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7547

020005 Title            ="Spark Advance ALDL "

020011 DescSize         =0x13

020010 Desc             ="ALDL Spark Advance"

020020 Units            ="Degrees"

020100 Address          =0x23

020200 Equation         =X/3,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x49

020005 Title            ="Stall Saver Spark Enable RPM Threshold 1"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x24

020200 Equation         =12.500000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4A

020005 Title            ="Stall Saver Spark Enable RPM Threshold 2"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x25

020200 Equation         =12.500000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4B

020005 Title            ="Stall Saver Spark Enable RPM Threshold 3"

020011 DescSize         =0x29

020010 Desc             ="Stall Saver Spark Enable RPM Threshold 3"

020020 Units            ="RPM"

020100 Address          =0x26

020200 Equation         =12.500000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4C

020005 Title            ="Stall Saver Spark Disable RPM Threshold"

020011 DescSize         =0x28

020010 Desc             ="Stall Saver Spark Disable RPM Threshold"

020020 Units            ="RPM"

020100 Address          =0x27

020200 Equation         =12.500000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2FFB

020005 Title            ="Stall Saver Spark Adder "

020011 DescSize         =0x2A

020010 Desc             ="Add To Spark Advance B L N D (VAL = 1 * )"

020020 Units            ="Degrees"

020100 Address          =0x28

020200 Equation         =0.351567 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6B77

020005 Title            ="If Cool Temp < Disable Hot Restart Spark Retard"

020011 DescSize         =0x41

020010 Desc             ="IF COOL < DISABLE HOT RESTART SPK RTD.\r\n; ((Temp + 40)* 256/192)"

020020 Units            ="Deg F"

020100 Address          =0x29

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1F2E

020005 Title            ="Hot Restart Spark Retard"

020011 DescSize         =0x26

020010 Desc             ="HOT RESTART SPARK RETARD, (Val/2.844)"

020020 Units            ="Degrees"

020100 Address          =0x2A

020200 Equation         =X*2.844,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x59FB

020005 Title            ="If Cool Temp >= Disable Cold Engine Spark Retard"

020011 DescSize         =0x3A

020010 Desc             ="IF COOL >= DISABLE cold engine SPK\r\n; ((Val+ 40)*256/192)"

020020 Units            ="Degrees F"

020100 Address          =0x2B

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x8

020005 Title            ="Coolant Temp Spark Correction Disable Temp"

020011 DescSize         =0x2B

020010 Desc             ="Coolant Temp Spark Correction Disable Temp"

020020 Units            ="Degrees F"

020100 Address          =0x2B

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7FBC

020005 Title            ="If Diff Cool Temp from Start >= disable cold spk"

020011 DescSize         =0x3E

020010 Desc             ="IF Diff. COOL FN START >= disable cold spk\r\n; (Val * 256/192)"

020020 Units            ="Degrees F"

020100 Address          =0x2C

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4A5C

020005 Title            ="Main Spark Table Min load Value"

020011 DescSize         =0x40

020010 Desc             ="Main Spark Table Min Load Value Main Spark Table ,  (32 gm/sec)"

020020 Units            ="Grs/sec"

020100 Address          =0x2D

020200 Equation         =X+32,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6856

020005 Title            ="Main Spark Table Min RPM Value"

020011 DescSize         =0x2E

020010 Desc             =" Main Spark Table Min RPM Value (400 RPM)	   "

020020 Units            ="RPM"

020100 Address          =0x2E

020200 Equation         =X*25/2,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x9

020005 Title            ="Spark Bias Degrees  (retards spark)"

020011 DescSize         =0x124

020010 Desc             ="Spark Bias Degrees  (retards spark) from                               Spark Correction Coolant Temp vs LV8                                    Spark Retard 20 Degress                                                                To allow for negetive spark advance numbers at higher temps  "

020020 Units            ="Degrees"

020100 Address          =0x10A

020200 Equation         =0.351567 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x34

020005 Title            ="Hiway Spark Min Coolant Temp "

020011 DescSize         =0x2C

020010 Desc             ="Minimum Coolant Temp for Highway Mode Spark"

020020 Units            ="Degrees F"

020100 Address          =0x17B

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x35

020005 Title            ="Hiway Spark Mode Max Load Value (LV8)"

020011 DescSize         =0x30

020010 Desc             ="Maximum Load Value (LV8) for Highway Spark Mode"

020020 Units            ="LV8"

020100 Address          =0x17C

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x36

020005 Title            ="Hiway Spark Mode Min RPM"

020011 DescSize         =0x23

020010 Desc             ="Minimum RPM for Highway Spark Mode"

020020 Units            ="RPM"

020100 Address          =0x17D

020200 Equation         =25.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x37

020005 Title            ="Hiway Spark Mode Enable Delay"

020011 DescSize         =0x20

020010 Desc             ="Highway Spark Mode Enable Delay"

020020 Units            ="Seconds"

020100 Address          =0x17E

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0xA

020005 Title            ="ESC Minimum RPM for (Knock Retard) Enable"

020011 DescSize         =0x41

020010 Desc             ="Above this RPM, the ECM will correct the timing to reduce knock "

020020 Units            ="RPM"

020100 Address          =0x1B9

020200 Equation         =12.500000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x9

%%END%%

%%CONSTANT%%

000002 UniqueID         =0xB

020005 Title            ="ESC Minimum MPH for (Knock Retard) Enable"

020011 DescSize         =0x42

020010 Desc             ="Above this speed the ECM will correct the timing to reduce knock."

020020 Units            ="MPH"

020100 Address          =0x1BA

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x8

%%END%%

%%CONSTANT%%

000002 UniqueID         =0xC

020005 Title            ="ESC Delta Coolant from Startup for (Knock) Enable"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x1BC

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x4

%%END%%

%%CONSTANT%%

000002 UniqueID         =0xD

020005 Title            ="ESC Min Coolant Temp for (Knock Retard) Enable"

020011 DescSize         =0x39

020010 Desc             ="Minimum Coolant Temp for Electronic Spark Control Enable"

020020 Units            ="Degrees F"

020100 Address          =0x1BB

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x4

%%END%%

%%CONSTANT%%

000002 UniqueID         =0xE

020005 Title            ="ESC If %TPS < X disabled (Knock Ret)"

020011 DescSize         =0x26

020010 Desc             ="If %TPS disabled ESC - Disable for X "

020020 Units            ="% TPS"

020100 Address          =0x1BE

020200 Equation         =0.390625 * X ,TH|0|0|0|0|

020210 DecimalPl        =0x1

020300 ALDLObj1         =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5609

020005 Title            ="ESC If TPS Volts < X disabled (Knock Ret)"

020011 DescSize         =0x26

020010 Desc             ="If %TPS disabled ESC - Disable for X "

020020 Units            ="TPS Volts"

020100 Address          =0x1BE

020200 Equation         =X/51,TH|0|0|0|0|

020300 ALDLObj1         =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0xF

020005 Title            ="ESC Max %TPS for Enable"

020011 DescSize         =0x24

020010 Desc             ="Maximum %TPS for ESC (Knock) Enable"

020020 Units            ="% TPS"

020100 Address          =0x1BD

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x1

020300 ALDLObj1         =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5473

020005 Title            ="ESC Max TPS Volts for Enable"

020011 DescSize         =0x29

020010 Desc             ="Maximum TPS Volts for ESC (Knock) Enable"

020020 Units            ="TPS Volts"

020100 Address          =0x1BD

020200 Equation         =X/51,TH|0|0|0|0|

020300 ALDLObj1         =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1B42

020005 Title            ="PE Max %TPS for Enable"

020011 DescSize         =0x22

020010 Desc             ="Maximum %TPS for Power Enrichment"

020020 Units            ="%"

020100 Address          =0x1F2

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x1

020300 ALDLObj1         =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x57BA

020005 Title            ="PE Max TPS Volts for Enable"

020011 DescSize         =0x27

020010 Desc             ="Maximum TPS Volts for Power Enrichment"

020020 Units            ="Volts"

020100 Address          =0x1F2

020200 Equation         =X/51,TH|0|0|0|0|

020300 ALDLObj1         =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0xECF

020005 Title            ="PE Max TPS Decimal counts for Enable"

020011 DescSize         =0x30

020010 Desc             ="Maximum TPS Decimal counts for Power Enrichment"

020020 Units            ="Decimal"

020100 Address          =0x1F2

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x27

020005 Title            ="MAF High Diag. Error Threshold"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Grams/sec"

020100 Address          =0x210

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2FFC

020005 Title            ="Use this when TPS > this for MAF Default"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="%"

020100 Address          =0x21D

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x537D

020005 Title            ="Gms/sec Scale factor, gms/sec/IAC Count"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Grams/sec"

020100 Address          =0x21F

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5896

020005 Title            ="5 Gms/Sec air flow offset for MAF default"

020011 DescSize         =0x2A

020010 Desc             ="4 Gms/Sec air flow offset for MAF default"

020020 Units            ="Grams/sec"

020100 Address          =0x220

020200 Equation         = X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6959

020005 Title            ="O2 sensor Lean Disable ERR 44"

020011 DescSize         =0x38

020010 Desc             ="If 02 Sensor >= 0.19v, then disable ERR 44, (VDC * 226)"

020020 Units            ="Mv"

020100 Address          =0x23A

020200 Equation         =X/226,TH|0|0|0|0|

020210 DecimalPl        =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x60C8

020005 Title            ="O2 sensor Lean Disable ERR 44 Decimal"

020011 DescSize         =0x38

020010 Desc             ="If 02 Sensor >= 0.19v, then disable ERR 44, (VDC * 226)"

020020 Units            ="Decimal"

020100 Address          =0x23A

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x46A9

020005 Title            ="Seconds to set ERR 44 "

020011 DescSize         =0x17

020010 Desc             ="Seconds to set ERR 44 "

020020 Units            ="Seconds"

020100 Address          =0x23B

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5E44

020005 Title            ="Forced Knock Test enable Temp"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x237

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x28

020005 Title            ="EGR Enable if TPS > X & EGR Duty Cycle<>0"

020011 DescSize         =0x1A

020010 Desc             ="Set to 100 to disable EGR"

020020 Units            ="%TPS"

020100 Address          =0x245

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x29

020005 Title            ="EGR Enable if TPS > X & EGR Duty Cycle=0"

020011 DescSize         =0x1A

020010 Desc             ="Set to 100 to disable EGR"

020020 Units            ="%TPS"

020100 Address          =0x246

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2A

020005 Title            ="EGR if MAT < X Disable"

020011 DescSize         =0x70

020010 Desc             ="Disable EGR if MAT < X                                                                Set to 304 to disable EGR"

020020 Units            ="Degrees F"

020100 Address          =0x247

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3668

020005 Title            ="CCP If LV8 > This, Don't Change DC"

020011 DescSize         =0x2E

020010 Desc             ="IF LV8 > This, Don't Change Cansiter Purge DC"

020020 Units            ="LV8"

020100 Address          =0x276

020200 Equation         = X ,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7E66

020005 Title            ="CCP Delay Between Purge Updates"

020011 DescSize         =0x2A

020010 Desc             ="Delay Between Purge Updates, (Sec's * 10)"

020020 Units            ="Seconds"

020100 Address          =0x277

020200 Equation         = X *10,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2216

020005 Title            ="CCP If Int >= this, Increase DC"

020011 DescSize         =0x2B

020010 Desc             ="If Int >= this, Increase Cansiter Purge DC"

020020 Units            ="Units"

020100 Address          =0x278

020200 Equation         = X ,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0xB93

020005 Title            ="CCP If Int < this, Decrease DC"

020011 DescSize         =0x2A

020010 Desc             ="If Int < this, Decrease Canister Purge DC"

020020 Units            ="Units"

020100 Address          =0x279

020200 Equation         = X ,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6054

020005 Title            ="CCP If Int >= to LC278 Add this to DC"

020011 DescSize         =0x31

020010 Desc             ="If Int >= to LC278 Add this to Canister Purge DC"

020020 Units            ="Units"

020100 Address          =0x27A

020200 Equation         = X ,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1053

020005 Title            ="CCP If Int < LC279 Subtract this from DC"

020011 DescSize         =0x34

020010 Desc             ="If Int < LC279 Subtract this from Canister Purge DC"

020020 Units            ="Units"

020100 Address          =0x27B

020200 Equation         = X ,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x47B2

020005 Title            ="?CCP No Canister Purge if DC <= this, & Low TPS"

020011 DescSize         =0x2B

020010 Desc             ="No Canister Purge if DC <= this, & Low TPS"

020020 Units            ="Units"

020100 Address          =0x27C

020200 Equation         = X ,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x69D5

020005 Title            ="?CCP Enable if >= MPH"

020011 DescSize         =0x24

020010 Desc             ="Enable CCP if >= MPH, (MPH * (16/5)"

020020 Units            ="Units"

020100 Address          =0x27D

020200 Equation         = X *16/5,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x213F

020005 Title            ="?CCP If TPS > = Then Enable "

020011 DescSize         =0x1B

020010 Desc             ="If TPS > = then enable CCP"

020020 Units            ="Units"

020100 Address          =0x27E

020200 Equation         = X *16/5,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4437

020005 Title            ="?CCP If TPS > = Then Enable "

020011 DescSize         =0x1B

020010 Desc             ="If TPS > = then enable CCP"

020020 Units            ="Units"

020100 Address          =0x27F

020200 Equation         = X *16/5,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0xF33

020005 Title            ="?CCP If TPS > = Then Enable "

020011 DescSize         =0x1B

020010 Desc             ="If TPS > = then enable CCP"

020020 Units            ="Units"

020100 Address          =0x280

020200 Equation         = X *16/5,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x1939

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4D

020005 Title            ="TCC Unlock Time"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Seconds"

020100 Address          =0x2A6

020200 Equation         =0.100000 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4E

020005 Title            ="TCC Min Lockup Coolant Temp "

020011 DescSize         =0x24

020010 Desc             ="TCC Minimum Coolant Temp for Lockup"

020020 Units            ="Degrees F"

020100 Address          =0x2A7

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4F

020005 Title            ="TCC Coast Release"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="MPH"

020100 Address          =0x2A8

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x50

020005 Title            ="TCC Low MPH Coast Load Limit"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="%TPS"

020100 Address          =0x2A9

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x51

020005 Title            ="TCC Lock Delay After 4-3 Downshift"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Seconds"

020100 Address          =0x2AA

020200 Equation         =0.100000 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x52

020005 Title            ="TCC Delay Before Lock in Low Gear"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Seconds"

020100 Address          =0x2AB

020200 Equation         =0.100000 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x53

020005 Title            ="TCC Delay Before Lock in High Gear"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Seconds"

020100 Address          =0x2AC

020200 Equation         =0.1 * X,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x54

020005 Title            ="TCC Lock Delay Bypass Speed"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="MPH"

020100 Address          =0x2AD

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x55

020005 Title            ="TCC Unlock Prevention Threshold"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="MPH"

020100 Address          =0x2AE

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x10

020005 Title            ="Fan 1 DC Table MPH Min"

020011 DescSize         =0x2E

020010 Desc             ="Mininmum MPH for using Fan 1 Duty Cycle Table"

020020 Units            ="MPH"

020100 Address          =0x309

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x11

020005 Title            ="Fan 1 Turn On A/C On < MPH"

020011 DescSize         =0x20

020010 Desc             ="MPH for Fan 1 Turn On if A/C On"

020020 Units            ="MPH"

020100 Address          =0x30A

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x8

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x12

020005 Title            ="Fan 1 Time On Min "

020011 DescSize         =0x16

020010 Desc             ="Minimum Fan 1 Time On"

020020 Units            ="Seconds"

020100 Address          =0x30B

020200 Equation         =0.200000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x13

020005 Title            ="Fan 1 DC if MPH < Min and A/C Pressure Hi"

020011 DescSize         =0x38

020010 Desc             ="Fan 1 Duty Cycle if MPH < Minimum and A/C Pressure High"

020020 Units            ="%DC"

020100 Address          =0x30C

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x14

020005 Title            ="Fan 1 Turn On (A/C Off)"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x30D

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x15

020005 Title            ="Fan 1 Turn Off (A/C Off)"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x30E

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x4

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x16

020005 Title            ="Fan 1 Turn On (A/C On)"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x30F

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x4

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x17

020005 Title            ="Fan 1 Turn Off (A/C On)"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x310

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x4

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3DAB

020005 Title            ="Air Pump to Divert to Open Air Min RPM"

020011 DescSize         =0x3B

020010 Desc             ="Air Injection Managment Divert if cont > 4000 RPM & time >"

020020 Units            ="RPM"

020100 Address          =0x31E

020200 Equation         =X*25,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7C1E

020005 Title            ="Air Pump to Divert to Open Air Min LV8 "

020011 DescSize         =0x30

020010 Desc             ="Air Injection Managment If LV8 < 25 then Divert"

020020 Units            ="Lv8"

020100 Address          =0x320

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x73F1

020005 Title            ="Air Pump to Divert to Open Air Max LV8 "

020011 DescSize         =0x38

020010 Desc             ="Air Injection Managment If LV8 > 100 & MPH > Divert Air"

020020 Units            ="Lv8"

020100 Address          =0x321

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1B03

020005 Title            ="Air Pump to Divert to Open Air Max MPH "

020011 DescSize         =0x38

020010 Desc             ="Air Injection Managment If > 60 MPH & LV8 > then divert"

020020 Units            ="MPH"

020100 Address          =0x322

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7E72

020005 Title            ="Air Pump Temp Enable If Cool T < X  then divert"

020011 DescSize         =0x27

020010 Desc             ="If Cool T < 14c, (58.5f) then divert\r\n"

020020 Units            ="Degrees F"

020100 Address          =0x324

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x46

020005 Title            ="AE Zero Async if Neg Diff TPS >"

020011 DescSize         =0x2A

020010 Desc             ="Zero Async Accel Enrich if Neg Diff TPS >"

020020 Units            ="%"

020100 Address          =0x328

020200 Equation         =0.390125 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x43

020005 Title            ="Trans TPS filter coef."

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            =""

020100 Address          =0x326

020200 Equation         =0.003906 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x47

020005 Title            ="AE Delta %TPS Min"

020011 DescSize         =0x24

020010 Desc             ="Minimum Delta %TPS For Accel Enrich"

020020 Units            ="%TPS"

020100 Address          =0x327

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x64FC

020005 Title            ="AE Delta Min TPS Volts"

020011 DescSize         =0x24

020010 Desc             ="Minimum Delta %TPS For Accel Enrich"

020020 Units            ="Volts"

020100 Address          =0x327

020200 Equation         = X/51,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x44

020005 Title            ="AE Async Pulse Width Minimum"

020011 DescSize         =0x33

020010 Desc             ="Accelleration Enrichment Async Pulse Width Minimum"

020020 Units            ="mSec"

020050 SizeInBits       =0x10

020100 Address          =0x329

020200 Equation         =0.015259 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x45

020005 Title            ="AE Async Pulse Width Maximum"

020011 DescSize         =0x33

020010 Desc             ="Accelleration Enrichment Async Pulse Width Maximum"

020020 Units            ="msec"

020050 SizeInBits       =0x10

020100 Address          =0x32B

020200 Equation         =0.015259 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x48

020005 Title            ="AE Delta LV8 Min"

020011 DescSize         =0x2F

020010 Desc             ="Minimum Delta LV8 For Accelleration Enrichment"

020020 Units            ="LV8"

020100 Address          =0x356

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5A2C

020005 Title            ="Throt Hi - Throt Lo % TPS"

020011 DescSize         =0x14

020010 Desc             ="Throt Hi - Throt Lo"

020020 Units            =" %TPS "

020100 Address          =0x3A3

020200 Equation         =0.390625 * X ,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7AB6

020005 Title            ="Throt Hi - Throt Lo TPS Volts"

020011 DescSize         =0x14

020010 Desc             ="Throt Hi - Throt Lo"

020020 Units            =" TPS Volts"

020100 Address          =0x3A3

020200 Equation         =X/51,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5468

020005 Title            ="Throt Hi - Throt Lo TPS Volts Dec scalar"

020011 DescSize         =0x14

020010 Desc             ="Throt Hi - Throt Lo"

020020 Units            =" Decimal"

020100 Address          =0x3A3

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x78B5

020005 Title            ="Min Throttle Position % TPS"

020011 DescSize         =0x1D

020010 Desc             ="13.7% Min TPS Throttle Posit"

020020 Units            =" %TPS"

020100 Address          =0x3A4

020200 Equation         =0.390625 * X,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3C73

020005 Title            ="Min Throttle Position  TPS Volts"

020011 DescSize         =0x20

020010 Desc             ="13.7% Min TPS Throttle Position"

020020 Units            =" TPS Volts"

020100 Address          =0x3A4

020200 Equation         =X/51,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x91C

020005 Title            ="O2 Initial Valve at Startup"

020011 DescSize         =0xD1

020010 Desc             ="12.5 msec o2 Filter initial value\r\nWHEN ENG NOT RUNNING, (0.398 VDC)\r\nCALIB = Arg * 256             X/226*1000 = mv                          Hex 66 Dec 102 / 226 * 1000 = 451 mv                               "

020020 Units            ="Mv"

020100 Address          =0x3A9

020200 Equation         =X/226*1000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2772

020005 Title            ="Min IBPW "

020011 DescSize         =0xC

020010 Desc             ="MIN BASE PW"

020020 Units            ="msec"

020100 Address          =0x3AC

020200 Equation         =X/65.536,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1461

020005 Title            ="Default IBPW"

020011 DescSize         =0x2E

020010 Desc             ="Default Pulse Width if calculated PW is <= to"

020020 Units            ="msec"

020100 Address          =0x3AE

020200 Equation         =X/65.536,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2B

020005 Title            ="Speed Limiter - Stop Fuel"

020011 DescSize         =0x3A

020010 Desc             ="Speed at which fuel will be cut.                         "

020020 Units            ="MPH"

020100 Address          =0x3AF

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020205 OutType          =0x2

020300 ALDLObj1         =0x8

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2D

020005 Title            ="Rev Limit - Stop Fuel"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020050 SizeInBits       =0x10

020100 Address          =0x3B0

020200 Equation         =983600.000000/X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x9

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2C

020005 Title            ="Speed Limiter - Re-start Fuel"

020011 DescSize         =0x61

020010 Desc             ="Speed at which fuel will restart. Must be less than the fuel cut speed.                         "

020020 Units            ="MPH"

020100 Address          =0x3B2

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x8

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2E

020005 Title            ="Rev Limit - Re-start Fuel"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020050 SizeInBits       =0x10

020100 Address          =0x3B3

020200 Equation         =983600.000000/X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x9

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x42

000100 Cat0ID           =0x1

020005 Title            ="Air/Fuel Ratio Stoichiometric "

020011 DescSize         =0x1E

020010 Desc             ="Stoichiometric Air/Fuel Ratio"

020020 Units            ="AFR"

020050 SizeInBits       =0x10

020100 Address          =0x3D5

020200 Equation         =6553.600000/X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x26F6

020005 Title            ="Injector Size Double Fire "

020011 DescSize         =0x1E

020010 Desc             ="Double Fire FI Size @ 40 psig"

020020 Units            ="Lbs/Hr"

020050 SizeInBits       =0x10

020100 Address          =0x3D7

020200 Equation         =10157.6/X,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2451

020005 Title            ="Injector Size Double Fire Dec"

020011 DescSize         =0x1E

020010 Desc             ="Double Fire FI Size @ 40 psig"

020020 Units            ="Decimal"

020050 SizeInBits       =0x10

020100 Address          =0x3D7

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7676

020005 Title            ="Injector Size Single Fire "

020011 DescSize         =0x1E

020010 Desc             ="Single Fire FI Size @ 40 psig"

020020 Units            ="Lbs/Hr"

020050 SizeInBits       =0x10

020100 Address          =0x3D9

020200 Equation         =20318.8/X,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2E28

020005 Title            ="Injector Size Single Fire Dec"

020011 DescSize         =0x1E

020010 Desc             ="Single Fire FI Size @ 40 psig"

020020 Units            ="Dec"

020050 SizeInBits       =0x10

020100 Address          =0x3D9

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6B25

020005 Title            ="Scalar for LC3FD"

020011 DescSize         =0x11

020010 Desc             ="SCALER FOR LC3FD"

020020 Units            ="Scale Factor"

020100 Address          =0x3DB

020200 Equation         =X/2,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x247B

020005 Title            ="Scalar for LC3FD Decimal"

020011 DescSize         =0x11

020010 Desc             ="SCALER FOR LC3FD"

020020 Units            ="Decimal"

020100 Address          =0x3DB

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7F09

020005 Title            ="AFR Bias"

020011 DescSize         =0x65

020010 Desc             ="AFR BIAS, USED TO SET LEAN LIMIT			        ARG = 256 - VALUE						20:1 AFR							14.7/20 x 256 = 189"

020020 Units            ="AFR"

020100 Address          =0x419

020200 Equation         =14.7/X*256,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7294

020005 Title            ="AFR Bias Decimal"

020011 DescSize         =0x65

020010 Desc             ="AFR BIAS, USED TO SET LEAN LIMIT			        ARG = 256 - VALUE						20:1 AFR							14.7/20 x 256 = 189"

020020 Units            ="Decimal"

020100 Address          =0x419

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2F

020005 Title            ="Hiway Fuel Mode Max Ld Value (LV8) to Remain"

020011 DescSize         =0x38

020010 Desc             ="Maximum Load Value (LV8) to Remain In Highway Fuel Mode"

020020 Units            ="LV8"

020100 Address          =0x43C

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0xC

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x30

020005 Title            ="Hiway Fuel Mode Min Coolant Temp"

020011 DescSize         =0x2B

020010 Desc             ="Minimum Coolant Temp for Highway Fuel Mode"

020020 Units            ="Degrees F"

020100 Address          =0x43D

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x4

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x31

020005 Title            ="Hiway Fuel Mode Enable Delay"

020011 DescSize         =0x1F

020010 Desc             ="Highway Fuel Mode Enable Delay"

020020 Units            ="Seconds"

020100 Address          =0x43E

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x32

020005 Title            ="Hiway Fuel Mode Max Time "

020011 DescSize         =0x22

020010 Desc             ="Maximum Time in Highway Fuel Mode"

020020 Units            ="min"

020100 Address          =0x43F

020200 Equation         =X/60,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x33

020005 Title            ="Hiway Fuel Mode Min Speed MPH to Enable "

020011 DescSize         =0x2E

020010 Desc             ="Minimum Speed MPH to Enable Highway Fuel Mode"

020020 Units            ="MPH"

020100 Address          =0x440

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x8

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1A

020005 Title            ="CL Temp Deg F Enable "

020011 DescSize         =0x2E

020010 Desc             ="Minimum engine temp before O2 sensor is used."

020020 Units            ="Degrees F"

020100 Address          =0x445

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1E

020005 Title            ="CL Hot Timer Temp Threshold"

020011 DescSize         =0x113

020010 Desc             ="Hot Closed Loop Timer Temp Threshold  If the engine is above this, it is considered \"cold\" and the timer value for hot closed loop will be used to determine when the car will enter closed loop.If the car is between the hot and cold temp threshholds, it is considered \"warm\"."

020020 Units            ="Degrees F"

020100 Address          =0x446

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1F

020005 Title            ="CL Cold Timer Temp Threshold"

020011 DescSize         =0x11B

020010 Desc             ="Cold Closed Loop Timer Temp Threshold  If the engine is at or below this, it is considered \"cold\" and the timer value for cold closed loop will be used to determine when the car will enter closed loop.If the car is between the hot and cold temp threshholds, it is considered \"warm\"."

020020 Units            ="Degrees F"

020100 Address          =0x447

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1D

020005 Title            ="CL Cold O2 Timer"

020011 DescSize         =0x1A

020010 Desc             ="Cold O2 Closed Loop Timer"

020020 Units            ="Seconds"

020100 Address          =0x448

020200 Equation         =2.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1C

020005 Title            ="CL Warm O2 Timer"

020011 DescSize         =0x1A

020010 Desc             ="Warm O2 Closed Loop Timer"

020020 Units            ="Seconds"

020100 Address          =0x449

020200 Equation         =2.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1B

020005 Title            ="CL Hot O2  Timer"

020011 DescSize         =0x19

020010 Desc             ="Hot O2 Closed Loop Timer"

020020 Units            ="Seconds"

020100 Address          =0x44A

020200 Equation         =2.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5BA0

020005 Title            ="If o2 Volts > then o2 ready"

020011 DescSize         =0x30

020010 Desc             ="699 mvdc, If o2 Volts > Threshold then o2 ready"

020020 Units            ="Mv"

020100 Address          =0x44B

020200 Equation         =X/226*1000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3AE5

020005 Title            ="If o2 Volts < then o2 ready"

020011 DescSize         =0x23

020010 Desc             ="If o2 < 0.199 VDC then o2 is ready"

020020 Units            ="Mv"

020100 Address          =0x44C

020200 Equation         =X/.226,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x45C2

020005 Title            ="If o2 in window  for 10 Sec, o2 state Not Ready"

020011 DescSize         =0x48

020010 Desc             ="If o2 in window formed by LC55E & LC55F for 10 Sec, o2 state\r\nNOT READY"

020020 Units            ="Seconds"

020100 Address          =0x44D

020200 Equation         =X/5,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5528

020005 Title            ="Intergrator Min Value"

020011 DescSize         =0x31

020010 Desc             ="Intergrator Minumum Value Closed Loop Correction"

020020 Units            ="steps"

020100 Address          =0x44E

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x401C

020005 Title            ="Intergrator Max value"

020011 DescSize         =0x32

020010 Desc             ="Intergrator  Maximum Value Closed Loop Correction"

020020 Units            ="steps"

020100 Address          =0x44F

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6537

020005 Title            ="O2 EECC Closed Loop Gain Factor"

020011 DescSize         =0x26

020010 Desc             ="O2 EECC Closed Loop Gain Factor x/256"

020020 Units            ="Multiplier"

020100 Address          =0x452

020200 Equation         =X/256,TH|0|0|0|0|

020210 DecimalPl        =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x4144

020005 Title            ="O2 Fast mv offset"

020011 DescSize         =0x3F

020010 Desc             ="Diff Value to Make up Rich/Lean Window for Fast o2.(VDC * 226)"

020020 Units            ="mv"

020100 Address          =0x453

020200 Equation         =X/226*1000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x284F

020005 Title            ="O2 Divert Air inject mv offset "

020011 DescSize         =0x4C

020010 Desc             ="Diff Value to Make up Rich/Lean Window When Air div.(VDC *\r\n226) subtracted"

020020 Units            ="mv"

020100 Address          =0x454

020200 Equation         =X/226*1000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x759B

020005 Title            ="O2 Positive Error Gain Factor"

020011 DescSize         =0x24

020010 Desc             ="O2 Positive Error Gain Factor x/256"

020020 Units            ="Multiplier"

020100 Address          =0x456

020200 Equation         =X/256,TH|0|0|0|0|

020210 DecimalPl        =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x64

020005 Title            ="BLM Cell RPM Boundary #1"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x4C7

020200 Equation         =25.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x65

020005 Title            ="BLM Cell RPM Boundary #2"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x4C8

020200 Equation         =25.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x66

020005 Title            ="BLM Cell RPM Boundary #3"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x4C9

020200 Equation         =25.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x67

020005 Title            ="BLM Cell Airflow Boundary #1"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Grams/sec"

020100 Address          =0x4CA

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x68

020005 Title            ="BLM Cell Airflow Boundary #2"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Grams/sec"

020100 Address          =0x4CB

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x69

020005 Title            ="BLM Cell Airflow Boundary #3"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Grams/sec"

020100 Address          =0x4CC

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6A

020005 Title            ="BLM Minimum Update Temp"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x4CF

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6B

020005 Title            ="BLM Maximum Update Temp"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x4D0

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6C

020005 Title            ="BLM LV8 to Update Minimum "

020011 DescSize         =0x1A

020010 Desc             ="Minimum LV8 to Update BLM"

020020 Units            ="LV8"

020100 Address          =0x4D1

020200 Equation         =0.048214 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6D

020005 Title            ="BLM Minimum Update Rate"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Seconds"

020100 Address          =0x4D2

020200 Equation         =0.048214 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6E

020005 Title            ="BLM Update Amount"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Interval"

020100 Address          =0x4D3

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x6F

020005 Title            ="BLM Allowable Maximum"

020011 DescSize         =0x16

020010 Desc             ="Maximum Allowable BLM"

020020 Units            ="BLM"

020100 Address          =0x4D4

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x70

020005 Title            ="BLM Allowable Minimum"

020011 DescSize         =0x16

020010 Desc             ="Minimum Allowable BLM"

020020 Units            ="BLM"

020100 Address          =0x4D5

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x71

020005 Title            ="Minimum Diff. Between BLM and INT"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="BLM"

020100 Address          =0x4D6

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x72

020005 Title            ="Maximum Diff. Between BLM and INT"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="BLM"

020100 Address          =0x4D7

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x60

020005 Title            ="Decel Enlean Enable Delta TPS Threshold"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="%TPS"

020100 Address          =0x4E6

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5E

020005 Title            ="Decel Enlean Enable if LV8 <= X"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="LV8"

020100 Address          =0x4E3

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5F

020005 Title            ="Decel Enlean Enable if RPM above"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x4E4

020200 Equation         =25.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x61

020005 Title            ="Decel Enlean Enable Delta LV8 Threshold"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="LV8"

020100 Address          =0x4E7

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x62

020005 Title            ="Decel Enlean BPW Factor (Low DRP's)"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Factor"

020100 Address          =0x4EA

020200 Equation         =0.007813 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x63

020005 Title            ="Decel Enlean BPW Factor (High DRP's)"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Factor"

020100 Address          =0x4EB

020200 Equation         =0.007813 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x38

000100 Cat0ID           =0x1

020005 Title            ="DFCO Enable if RPM above "

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x4EC

020200 Equation         =25.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x9

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3C

000100 Cat0ID           =0x1

020005 Title            ="DFCO Enable if LV8 < X"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="LV8"

020100 Address          =0x4EE

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0xC

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3D

000100 Cat0ID           =0x1

020005 Title            ="DFCO Remain if LV8 < X"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="LV8"

020100 Address          =0x4EF

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0xC

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3E

000100 Cat0ID           =0x1

020005 Title            ="DFCO Max RPM Decrease/12.5 ms"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x4F0

020200 Equation         =12.500000 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x40

000100 Cat0ID           =0x1

020005 Title            ="DFCO Disable Low VSS Speed Limit"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="MPH"

020100 Address          =0x4F4

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3F

000100 Cat0ID           =0x1

020005 Title            ="DFCO Enable if %TPS < X "

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="%TPS"

020100 Address          =0x4F2

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x1

020300 ALDLObj1         =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2683

000100 Cat0ID           =0x1

020005 Title            ="DFCO Enable if TPS Volts < X "

020011 DescSize         =0x1E

020010 Desc             ="DFCO Enable if TPS Volts < X "

020020 Units            ="Volts TPS"

020100 Address          =0x4F2

020200 Equation         =X/51,TH|0|0|0|0|

020300 ALDLObj1         =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x39

000100 Cat0ID           =0x1

020005 Title            ="DFCO Remain if RPM above"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x4ED

020200 Equation         =25.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x9

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3B

000100 Cat0ID           =0x1

020005 Title            ="DFCO Enabled if conditions met for X"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Seconds"

020100 Address          =0x4F1

020200 Equation         =0.012500 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3A

000100 Cat0ID           =0x1

020005 Title            ="DFCO Disable if Temp Less Than"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x4F3

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0x4

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x41

000100 Cat0ID           =0x1

020005 Title            ="DFCO Consecutive Min Delay Time"

020011 DescSize         =0x24

020010 Desc             ="DFCO Consecutive Minimum Delay Time"

020020 Units            ="Seconds"

020050 SizeInBits       =0x10

020100 Address          =0x4FA

020200 Equation         =0.012500 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x18

020005 Title            ="PE LV8 to Enable Min"

020011 DescSize         =0x27

020010 Desc             ="Minimum LV8 to Enable Power Enrichment"

020020 Units            ="LV8"

020100 Address          =0x4FD

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0xC

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x69DF

020005 Title            ="PE LV8 Hysteresis"

020011 DescSize         =0x20

020010 Desc             ="Power Enrichment LV8 Hysteresis"

020020 Units            ="LV8"

020100 Address          =0x4FE

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

020300 ALDLObj1         =0xC

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3D67

020005 Title            ="PE %TPS Hysteresis"

020011 DescSize         =0x19

020010 Desc             ="%TPS HYST FOR PWR ENRICH"

020020 Units            =""

020100 Address          =0x4FF

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x65FF

020005 Title            ="PE TPS Volts Hysteresis"

020011 DescSize         =0x18

020010 Desc             ="PE TPS Volts Hysteresis"

020020 Units            =""

020100 Address          =0x4FF

020200 Equation         =X/51,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x19

020005 Title            ="PE Temp to Use Hot Coolant PE %TPS Min"

020011 DescSize         =0x36

020010 Desc             ="Minimum Temp to Use Hot Coolant Power Enrichment %TPS"

020020 Units            ="Degrees F"

020100 Address          =0x500

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1EC

020005 Title            ="Load Variable for Scaling LV8 Scale Factors"

020011 DescSize         =0x2E

020010 Desc             ="Load Variable for Scaling LV8 Scale Factors\r\n"

020020 Units            ="Scale Factor"

020100 Address          =0x5B0

020200 Equation         =X/64,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x1A92

020005 Title            ="Load Variable for Scaling LV8 Scale Factors Dec"

020011 DescSize         =0x32

020010 Desc             ="Load Variable for Scaling LV8 Scale Factors Dec\r\n"

020020 Units            ="Decimal"

020100 Address          =0x5B0

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x22

020005 Title            ="MAF Table #1 Scalar"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            =""

020100 Address          =0x5B3

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x23

020005 Title            ="MAF Table #2 Scalar"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            =""

020100 Address          =0x5BE

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x24

020005 Title            ="MAF Table #3 Scalar"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            =""

020100 Address          =0x5C9

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x25

020005 Title            ="MAF Table #4 Scalar"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            =""

020100 Address          =0x5D4

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x26

020005 Title            ="MAF Table #5 Scalar"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            =""

020100 Address          =0x5DF

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x57D3

020005 Title            ="MAF Table #6 Scalar"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            =""

020100 Address          =0x5EA

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x32F

020005 Title            ="MAF Multiplier for Sliding Air Flow Filter"

020011 DescSize         =0x2B

020010 Desc             ="Maf Multiplier for Sliding Air Flow Filter"

020020 Units            =""

020100 Address          =0x5FD

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x8B5

020005 Title            ="MAF Min Allowable Flow Grs/Sec"

020011 DescSize         =0x1E

020010 Desc             ="3 gms/Sec, Min Allowable Flow"

020020 Units            ="Grs/sec"

020100 Address          =0x5FE

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x56

020005 Title            ="Lo Speed Limit Lo Gear Lock To Unlock Transition"

020011 DescSize         =0x37

020010 Desc             ="Low Speed Limit For Low Gear Lock To Unlock Transition"

020020 Units            ="MPH"

020100 Address          =0x2B2

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x57

020005 Title            ="Lo Speed Limit Lo Gear Unlock To Lock Transition"

020011 DescSize         =0x37

020010 Desc             ="Low Speed Limit For Low Gear Unlock To Lock Transition"

020020 Units            ="MPH"

020100 Address          =0x2B0

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x58

020005 Title            ="Hi Speed Limit Lo Gear Lock To Unlock Transition"

020011 DescSize         =0x38

020010 Desc             ="High Speed Limit For Low Gear Lock To Unlock Transition"

020020 Units            ="MPH"

020100 Address          =0x2B1

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x59

020005 Title            ="Hi Speed Limit Lo Gear Unlock To Lock Transition"

020011 DescSize         =0x38

020010 Desc             ="High Speed Limit For Low Gear Unlock To Lock Transition"

020020 Units            ="MPH"

020100 Address          =0x2B3

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5A

020005 Title            ="Lo Speed Limit Hi Gear Lock To Unlock Transition"

020011 DescSize         =0x38

020010 Desc             ="Low Speed Limit For High Gear Lock To Unlock Transition"

020020 Units            ="MPH"

020100 Address          =0x2CE

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5B

020005 Title            ="Lo Speed Limit Hi Gear Unlock To Lock Transition"

020011 DescSize         =0x38

020010 Desc             ="Low Speed Limit For High Gear Unlock To Lock Transition"

020020 Units            ="MPH"

020100 Address          =0x2CC

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5C

020005 Title            ="Hi Speed Limit Hi Gear Unlock To Lock Transition"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="MPH"

020100 Address          =0x2CD

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5D

020005 Title            ="Hi Speed Limit Hi Gear Lock To Unlock Transition"

020011 DescSize         =0x39

020010 Desc             ="High Speed Limit For High Gear Lock To Unlock Transition"

020020 Units            ="MPH"

020100 Address          =0x2CF

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x73

000100 Cat0ID           =0x3

020005 Title            ="IAC RPM Start Up Position"

020011 DescSize         =0x1B

020010 Desc             ="Idle RPM Start Up Position"

020020 Units            ="Steps"

020100 Address          =0x527

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x74

020005 Title            ="IAC Steps Added to Warm Park Position if A/C On"

020011 DescSize         =0x30

020010 Desc             ="IAC Steps Added to Warm Park Position if A/C On"

020020 Units            ="Steps"

020100 Address          =0x528

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x75

020005 Title            ="IAC Steps Added for Fan Anticipate"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Steps"

020100 Address          =0x529

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x76

020005 Title            ="IAC Speed Adder Park/Neutral"

020011 DescSize         =0x1E

020010 Desc             ="Idle Speed Adder Park/Neutral"

020020 Units            ="RPM"

020100 Address          =0x52B

020200 Equation         =12.500000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x77

020005 Title            ="IAC Speed Start-up Delay"

020011 DescSize         =0x1A

020010 Desc             ="Idle Speed Start-up Delay"

020020 Units            ="msec"

020100 Address          =0x52C

020200 Equation         =8.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x78

020005 Title            ="IAC Speed Adder A/C Enabled"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x52D

020200 Equation         =12.500000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x79

020005 Title            ="IAC Target Speed Deadband For Park/Neutral"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x53F

020200 Equation         =12.500000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7A

020005 Title            ="IAC Target Speed Deadband For Drive"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x540

020200 Equation         =12.500000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7F

020005 Title            ="IAC PID if MPH is GT Enable "

020011 DescSize         =0x1D

020010 Desc             ="IAC PID if MPH is GT Enable "

020020 Units            ="MPH"

020100 Address          =0x545

020200 Equation         =0.312500 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x62D2

020005 Title            ="T/F TPS Follower Slope Gain"

020011 DescSize         =0x2B

020010 Desc             ="T/F TPS Follower Slope Gain 150 STEPS/%TPS"

020020 Units            ="Gain"

020100 Address          =0x559

020200 Equation         =X/1.28,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x366

020005 Title            ="T/F BETWEEN STEP'S IN PARK/NEUT "

020011 DescSize         =0x2F

020010 Desc             ="T/F BETWEEN STEP'S IN PK/NEUT  arg = msec/6.25"

020020 Units            ="MSec"

020100 Address          =0x555

020200 Equation         =X*6.25,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x5A24

020005 Title            ="T/F BETWEEN STEP'S IN DRIVE"

020011 DescSize         =0x2C

020010 Desc             ="T/F BETWEEN STEP'S IN DRIVE arg = msec/6.25"

020020 Units            ="MSec"

020100 Address          =0x556

020200 Equation         =X*6.25,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x359D

020005 Title            ="T/F Max Steps in Drive"

020011 DescSize         =0x17

020010 Desc             ="T/F Max Steps in Drive"

020020 Units            ="Steps"

020100 Address          =0x55A

020200 Equation         =X,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3132

020005 Title            ="T/F Park/Neutral Steps/Drive Steps TF MULT "

020011 DescSize         =0x39

020010 Desc             ="T/F Park/Neutral Steps/Drive Steps TF MULT 0.996 (X*256)"

020020 Units            ="Multiplier"

020100 Address          =0x55B

020200 Equation         =X/256,TH|0|0|0|0|

020210 DecimalPl        =0x3

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7E

020005 Title            ="A/C Disable Above %TPS"

020011 DescSize         =0x17

020010 Desc             ="Disable A/C Above %TPS"

020020 Units            ="%TPS"

020100 Address          =0x564

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x81

020005 Title            ="A/C Disable Above Delta %TPS"

020011 DescSize         =0x1D

020010 Desc             ="Disable A/C Above Delta %TPS"

020020 Units            ="%TPS"

020100 Address          =0x565

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x2669

020005 Title            ="A/C Disable Above Delta TPS Volts"

020011 DescSize         =0x22

020010 Desc             ="Disable A/C Above Delta TPS Volts"

020020 Units            ="TPS Volts"

020100 Address          =0x565

020200 Equation         = X/51,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x82

020005 Title            ="A/C Disable Above Coolant Temp"

020011 DescSize         =0x1F

020010 Desc             ="Disable A/C Above Coolant Temp"

020020 Units            ="Degrees F"

020100 Address          =0x569

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x80

020005 Title            ="IAC Steps Added to T/F in Drive"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Steps"

020100 Address          =0x57A

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7B

020005 Title            ="IAC Default Keep Alive Motor Steps"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Steps"

020100 Address          =0x597

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7C

020005 Title            ="IAC Minimum Keep Alive Learning Temp"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x598

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7D

020005 Title            ="IAC Maximum Keep Alive Learning Temp"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x599

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x83

020005 Title            ="Shift Light On Delay Modifier"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Seconds"

020100 Address          =0x303

020200 Equation         =0.100000 * X + 0.000000,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x84

020005 Title            ="Minimum LV8 for Shift Light On"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="LV8"

020100 Address          =0x2F0

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x85

020005 Title            ="Minimum %TPS for Shift Light On"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="%TPS"

020100 Address          =0x2EF

020200 Equation         =0.390625 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x86

020005 Title            ="Minimum Temp for Shift Light Enable"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="Degrees F"

020100 Address          =0x2ED

020200 Equation         =1.350000 * X + -40.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x87

020005 Title            ="Minimum MPH for Shift Light Enable"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="MPH"

020100 Address          =0x2EE

020200 Equation         =1.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x88

020005 Title            ="Minimum RPM for Shift Light Enable"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x2F1

020200 Equation         =25.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x89

020005 Title            ="Force Shift Light On Above RPM (Not in 5th)"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x2F2

020200 Equation         =25.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x8A

020005 Title            ="Force Shift Light Off Above RPM"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x2FB

020200 Equation         =25.000000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x8B

020005 Title            ="Force Shift Light Off Below RPM"

020011 DescSize         =0x1

020010 Desc             =""

020020 Units            ="RPM"

020100 Address          =0x2FA

020200 Equation         =12.500000 * X + 0.000000,TH|0|0|0|0|

020210 DecimalPl        =0x0

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x7217

020005 Title            ="Gals/Hr Inj flow rate (for MPG display)"

020011 DescSize         =0x42

020010 Desc             ="Gals/Hr Inj flow rate\r\nNum = Arg * 32, (3.8125 G/Hr)      x/5.305"

020020 Units            ="K"

020100 Address          =0x620

020200 Equation         =X/32,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x3230

020005 Title            ="Gals/Hr Inj flow rate (for MPG display) 39# FP"

020011 DescSize         =0x3A

020010 Desc             ="Gals/Hr Inj flow rate\r\nNum = Arg * 32, (3.8125 G/Hr)     "

020020 Units            ="#/hr"

020100 Address          =0x620

020200 Equation         =X/32*6.2449979,TH|0|0|0|0|

%%END%%

%%CONSTANT%%

000002 UniqueID         =0x37DA

020005 Title            ="Gals/Hr Inj flow rate (for MPG display) dec"

020011 DescSize         =0x41

020010 Desc             ="Gals/Hr Inj flow rate\r\nNum = Arg * 32, (3.8125 G/Hr)        32/X"

020020 Units            =""

020100 Address          =0x620

020200 Equation         =X,TH|0|0|0|0|

020205 OutType          =0x2

%%END%%

%%FLAG%%

000002 UniqueID         =0x8C

030005 Title            ="Use Filter for Air Flow"

030011 DescSize         =0x39

030010 Desc             ="Bit 7 = USE FILTER FOR AIR FLOW                         "

030100 Address          =0x14

030200 BitNumber        =0x7

%%END%%

%%FLAG%%

000002 UniqueID         =0x99

030005 Title            ="If Set - Use 2 Air Valves (1 if not set)"

030011 DescSize         =0x2D

030010 Desc             ="2 = Set = 2 AiR VALVES, Not Set = 1 VALVE   "

030100 Address          =0x15

030200 BitNumber        =0x2

%%END%%

%%FLAG%%

000002 UniqueID         =0xA2

030005 Title            ="Reset Corr/Init of Auto Enrich"

030011 DescSize         =0x2B

030010 Desc             ="1 = RESET CORR/INIT OF AUTO ENRICH        "

030100 Address          =0x16

030200 BitNumber        =0x1

%%END%%

%%FLAG%%

000002 UniqueID         =0x9E

030005 Title            ="Reset INT w/ BLM change"

030011 DescSize         =0x24

030010 Desc             ="5 = INT RESET WHEN B.LRN CELL CHNGE"

030100 Address          =0x16

030200 BitNumber        =0x5

%%END%%

%%FLAG%%

000002 UniqueID         =0x9D

030005 Title            ="Use IAC steps instead of desired idle speed"

030011 DescSize         =0xE5

030010 Desc             ="6 = CALC BASE INJ PW, NOT TBL   ????                                           Instead it uses the IAC counts vs CTS                                         set = rpm vs coolant temp                       unset IAC counts vs CTS"

030100 Address          =0x16

030200 BitNumber        =0x6

%%END%%

%%FLAG%%

000002 UniqueID         =0xAB

030005 Title            ="Use LV8 for BLM not DispFlow"

030011 DescSize         =0x24

030010 Desc             ="0 = Use LD VAL FOR BLM NOT DISPFLOW"

030100 Address          =0x17

030200 BitNumber        =0x0

%%END%%

%%FLAG%%

000002 UniqueID         =0xA9

030005 Title            ="4th Gear Hwy Mode Spark On"

030011 DescSize         =0x25

030010 Desc             ="2 = 4th GEAR HWY MODE SPARK ADV ON\r\n"

030100 Address          =0x17

030200 BitNumber        =0x2

%%END%%

%%FLAG%%

000002 UniqueID         =0xA8

030005 Title            ="If CCP Chg Force INT to 128"

030011 DescSize         =0x23

030010 Desc             ="3 = If CCP CHG, FORCE INT TO 128\r\n"

030100 Address          =0x17

030200 BitNumber        =0x3

%%END%%

%%FLAG%%

000002 UniqueID         =0xA7

030005 Title            ="TCC Locked Hwy Mode Spark Check On"

030011 DescSize         =0x23

030010 Desc             ="4 = TCC LK'ED HWY MODE SPARK CK ON"

030100 Address          =0x17

030200 BitNumber        =0x4

%%END%%

%%FLAG%%

000002 UniqueID         =0xA6

030005 Title            ="Cooling Fan N/O Req Input"

030011 DescSize         =0x1F

030010 Desc             ="5 = N.O. COOLING FAN REQ INPUT"

030100 Address          =0x17

030200 BitNumber        =0x5

%%END%%

%%FLAG%%

000002 UniqueID         =0xA5

030005 Title            ="Use Power Streeing Pressure Switch"

030011 DescSize         =0x1E

030010 Desc             ="6 = Use PWR STEER PRESSURE SW"

030100 Address          =0x17

030200 BitNumber        =0x6

%%END%%

%%FLAG%%

000002 UniqueID         =0xA4

030005 Title            ="Use TCC Out to Control A/C Clutch"

030011 DescSize         =0x24

030010 Desc             ="7 = Use TCC OUT TO CNT'L A/C CLUTCH"

030100 Address          =0x17

030200 BitNumber        =0x7

%%END%%

%%TABLE%%

000002 UniqueID         =0xAC

000100 Cat0ID           =0x2

040005 Title            ="Spark Advance Table   RPM vs LV8"

040011 DescSize         =0x72

040010 Desc             ="Main Spark Advance table. Edit slowly and carefully. This table greatly effects driveability, power, and economy."

040100 Address          =0x30

040150 Flags            =0x30

040200 ZEq              =0.351567 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x2

040304 YOutType         =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =43.0000

040300 Rows             =0x11

040305 Cols             =0xC

040320 XUnits           ="LV8"

040325 YUnits           ="RPM"

040330 ZUnits           ="Degrees"

040350 XLabels          =32,48,64,80,96,112,128,144,160,176,192,208

040352 XLabelType       =0x2

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =400,600,800,1000,1200,1400,1600,1800,2000,2200,2400,2800,3200,3600,4000,4400,4800

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040400 XALDLObj         =0xC

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x2FE9

000100 Cat0ID           =0x2

040005 Title            ="Time Domain Correction to Spark (Latency) Decimal"

040011 DescSize         =0x8F

040010 Desc             ="Time Domain Correction to Spark  Decimal                                      This corrects the time lag of the large cap distributor (vette)."

040100 Address          =0xFC

040150 Flags            =0x30

040200 ZEq              =X ,TH|0|0|0|0|

040203 XOutType         =0x2

040304 YOutType         =0x2

040210 DecimalPl        =0x0

040230 RangeLow         =0.0000

040240 RangeHigh        =43.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="Dec"

040325 YUnits           ="RPM"

040330 ZUnits           ="Degrees"

040350 XLabels          =32

040352 XLabelType       =0x2

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =800,1200,1600,2000,2400,2800,3200,3600,4000,4400,4800,5200,5600,6000

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040400 XALDLObj         =0xC

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x1B00

000100 Cat0ID           =0x2

040005 Title            ="Time Domain Correction to Spark (Latency)"

040011 DescSize         =0x8E

040010 Desc             ="Time Domain Correction to Spark                                              This corrects the time lag of the large cap distributor (vette)."

040100 Address          =0xFC

040150 Flags            =0x30

040200 ZEq              =15.26 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x2

040304 YOutType         =0x2

040210 DecimalPl        =0x0

040230 RangeLow         =0.0000

040240 RangeHigh        =43.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="usec"

040325 YUnits           ="RPM"

040330 ZUnits           ="Degrees"

040350 XLabels          =32

040352 XLabelType       =0x2

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =800,1200,1600,2000,2400,2800,3200,3600,4000,4400,4800,5200,5600,6000

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040400 XALDLObj         =0xC

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xAD

000100 Cat0ID           =0x2

040005 Title            ="Spark Correction Coolant Temp vs LV8"

040011 DescSize         =0x25

040010 Desc             ="Spark Correction Coolant Temp vs LV8"

040100 Address          =0x10F

040150 Flags            =0x30

040200 ZEq              =0.351567 * (X - Y) + 0.000000,TH|0|0|0|0|,AD|10A|8|0|0|

040203 XOutType         =0x2

040304 YOutType         =0x2

040230 RangeLow         =-15.0000

040240 RangeHigh        =15.0000

040300 Rows             =0xC

040305 Cols             =0x9

040320 XUnits           ="LV8"

040325 YUnits           ="Deg F"

040330 ZUnits           ="Degrees"

040350 XLabels          =32,48,64,80,96,112,128,144,160

040352 XLabelType       =0x2

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =03,24,46,68,90,111,133,154,176,198,219,241

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040400 XALDLObj         =0xC

040425 YALDLObj         =0x4

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xC9

040005 Title            ="Spark Advance vs LV8 Highway Mode"

040011 DescSize         =0x23

040010 Desc             ="Highway Mode Spark Advance vs Load"

040100 Address          =0x17F

040200 ZEq              =0.351567 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x8

040305 Cols             =0x1

040320 XUnits           ="Degrees"

040325 YUnits           ="LV8"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =   32,   48,   64,   80,   96,  112,  128,  144

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xAE

000100 Cat0ID           =0x2

040005 Title            ="Spark Adv vs Coolant Temp Startup"

040011 DescSize         =0x2E

040010 Desc             ="Startup Spark Advance vs. Coolant Temperature"

040100 Address          =0x18D

040200 ZEq              =0.351567 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="Degrees"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40,-18,03,24,46,68,90,111,133,154,176,198,219,241

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xB1

040005 Title            ="ESC Attack Rate"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x1C2

040200 ZEq              =0.022500 * X + 0.000000,TH|0|0|0|0|

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x5

040305 Cols             =0x1

040320 XUnits           ="Deg per ms"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =400,1200,2000,3200,4800

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xB2

040005 Title            ="ESC Recovery Rate"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x1C7

040200 ZEq              =1.950000 * X + 0.000000,TH|0|0|0|0|

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x5

040305 Cols             =0x1

040320 XUnits           ="Percent/Sec"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =400,1200,2000,3200,4800

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xAF

000100 Cat0ID           =0x2

040005 Title            ="Startup Spark  Decay Delay Vs Coolant Temp"

040011 DescSize         =0x31

040010 Desc             ="Startup Spark Decay Delay Vs Coolant Temperature"

040100 Address          =0x19D

040200 ZEq              =X ,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="Injects"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40,-18,03,24,46,68,90,111,133,154,176,198,219,241

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xB0

000100 Cat0ID           =0x2

040005 Title            ="Startup Spark Decay Vs Coolant Temp"

040011 DescSize         =0x2B

040010 Desc             ="Startup Spark Decay Vs Coolant Temperature"

040100 Address          =0x1AB

040200 ZEq              =0.001373 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="Deg Spk"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40,-18,03,24,46,68,90,111,133,154,176,198,219,241

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x62DC

000100 Cat0ID           =0x1

040005 Title            ="Startup Enrichment Delay Vs Coolant Temp (Choke)"

040011 DescSize         =0x35

040010 Desc             ="After Start Enrichment Essentially a choke delay off"

040100 Address          =0x3ED

040200 ZEq              =0.390625 * X,TH|0|0|0|0|

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="Percent"

040325 YUnits           ="Deg F"

040330 ZUnits           ="% Change"

040350 XLabels          =0.00

040352 XLabelType       =0x1

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40.00,-18.00,3.00,24.00,46.00,68.00,90.00,111.00,133.00,154.00,176.00,198.00,219.00,241.00

040362 YLabelType       =0x1

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x198C

000100 Cat0ID           =0x1

040005 Title            ="Startup Enrichment Delay Vs Coolant Temp (Choke) Decimal"

040011 DescSize         =0x35

040010 Desc             ="After Start Enrichment Essentially a choke delay off"

040100 Address          =0x3ED

040200 ZEq              = X,TH|0|0|0|0|

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="Percent"

040325 YUnits           ="Deg F"

040330 ZUnits           ="% Change"

040350 XLabels          =0.00

040352 XLabelType       =0x1

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40.00,-18.00,3.00,24.00,46.00,68.00,90.00,111.00,133.00,154.00,176.00,198.00,219.00,241.00

040362 YLabelType       =0x1

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x26C1

040005 Title            ="Startup Enrichment Decay Delay Injects vs. Coolant Temp"

040011 DescSize         =0x71

040010 Desc             ="Number of injections before enrichment decays after startup. Startup Enrich Decay Delay vs. Coolant Temperature."

040100 Address          =0x3FD

040200 ZEq              =X * (X / 256),TH|0|0|0|0|,TH|0|0|0|0|

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="Injects"

040325 YUnits           ="Deg F"

040330 ZUnits           ="Injections"

040350 XLabels          =0.00

040352 XLabelType       =0x1

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40.00,18.00,3.00,24.00,46.00,68.00,90.00,111.00,133.00,154.00,176.00,198.00,219.00,241.00

040362 YLabelType       =0x1

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x18D6

040005 Title            ="Startup Enrichment Decay Delay % Change vs Coolant Temp"

040011 DescSize         =0x7E

040010 Desc             ="Start Up Enrichment Decay Delay % Change vs Coolant             Start Up Enrichment Decay Delay Amount vs Coolant Temperature"

040100 Address          =0x40B

040200 ZEq              =X/6.553,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040210 DecimalPl        =0x0

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="% Change"

040325 YUnits           ="Temp"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40,-18,3,24,46,68,90,111,133,154,176,198,219,241

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x3BD7

040005 Title            ="Startup Enrichment Decay Delay % Change vs Coolant Temp Decimal"

040011 DescSize         =0x7E

040010 Desc             ="Start Up Enrichment Decay Delay % Change vs Coolant             Start Up Enrichment Decay Delay Amount vs Coolant Temperature"

040100 Address          =0x40B

040200 ZEq              =X,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040210 DecimalPl        =0x0

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="% Change"

040325 YUnits           ="Temp"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40,-18,3,24,46,68,90,111,133,154,176,198,219,241

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xD4

040005 Title            ="Crank Fuel PW Multiplier vs Ref Pulse"

040011 DescSize         =0x2C

040010 Desc             ="Crank Fuel PW Multiplier vs Reference Pulse"

040100 Address          =0x38B

040200 ZEq              =0.003906 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x18

040305 Cols             =0x1

040320 XUnits           ="Multiplier"

040325 YUnits           ="Ref. Pulse"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =    1,    2,    3,    4,    5,    6,    7,    8,    9,   10,   11,   12,   13,   14,   15,   16,   17,   18,   19,   20,   21,   22,   23,   24

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xD5

040005 Title            ="Crank Fuel PW vs Coolant Temp"

040011 DescSize         =0x25

040010 Desc             ="Crank Fuel PW vs Coolant Temperature"

040100 Address          =0x373

040200 ZEq              =0.781300 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="PW(msec)"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  -40,  -18,    3,   25,   46,   68,   90,  111,  133,  154,  176,  198,  219,  241

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x4AF9

040005 Title            ="Crank Fuel PW vs Coolant Temp Decimal"

040011 DescSize         =0x25

040010 Desc             ="Crank Fuel PW vs Coolant Temperature"

040100 Address          =0x373

040200 ZEq              =X ,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="PW(msec)"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  -40,  -18,    3,   25,   46,   68,   90,  111,  133,  154,  176,  198,  219,  241

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xD6

040005 Title            ="Crank Fuel PW vs TPS"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x382

040200 ZEq              =0.015625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="Multiplier"

040325 YUnits           ="% TPS"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =    0, 12.5, 25.0, 37.5, 50.0, 62.5, 75.0, 87.5, 100

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xB6

040005 Title            ="Open Loop AFR % Chg Vs LV8"

040011 DescSize         =0x1E

040010 Desc             ="Open Loop AFR % Change Vs LV8"

040100 Address          =0x3DC

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="% Change to AFR"

040325 YUnits           ="LV8"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =    0,   16,   32,   48,   64,   80,   96,  112,  128,  144,  160,  176,  192,  208,  224,  240,  256

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x1ECE

040005 Title            ="Open Loop AFR % Chg Vs LV8 Decimal"

040011 DescSize         =0x26

040010 Desc             ="Open Loop AFR % Change Vs LV8 Decimal"

040100 Address          =0x3DC

040200 ZEq              =X,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="% Change to AFR"

040325 YUnits           ="LV8"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =    0,   16,   32,   48,   64,   80,   96,  112,  128,  144,  160,  176,  192,  208,  224,  240,  256

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xB7

040005 Title            ="Open Loop AFR % Chg vs Coolant Temp"

040011 DescSize         =0x2F

040010 Desc             ="Open Loop AFR Ratio % Change vs. Cooolant Temp"

040100 Address          =0x41A

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="% Change to AFR"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  -40,  -18,    3,   24,   46,   68,   90,  111,  133,  154,  176,  198,  219,  241,  262,  284,  306

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x2967

040005 Title            ="Open Loop AFR % Chg Offset vs Coolant Temp "

040011 DescSize         =0x36

040010 Desc             ="Open Loop AFR Ratio % Change Offset vs. Cooolant Temp"

040100 Address          =0x41A

040200 ZEq              =X/2.56 - 13.67,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="% Change to AFR"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  -40,  -18,    3,   24,   46,   68,   90,  111,  133,  154,  176,  198,  219,  241,  262,  284,  306

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x338

040005 Title            ="Open Loop AFR % Chg vs Coolant Temp Decimal"

040011 DescSize         =0x37

040010 Desc             ="Open Loop AFR Ratio % Change vs. Cooolant Temp Decimal"

040100 Address          =0x41A

040200 ZEq              = X,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="% Change to AFR"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  -40,  -18,    3,   24,   46,   68,   90,  111,  133,  154,  176,  198,  219,  241,  262,  284,  306

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xB3

040005 Title            ="PE Knock Retard Added vs RPM in PE Mode (Wot)"

040011 DescSize         =0x60

040010 Desc             ="Max Knock Retard Degrees when in Power Enrichment Mode  Maximum Knock Retard when in Wot vs RPM"

040100 Address          =0x1CC

040200 ZEq              =0.175800 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x8

040305 Cols             =0x1

040320 XUnits           ="Degrees"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =800,1600,2400,3200,4000,4800,5600,6400

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xBB

040005 Title            ="PE Min % TPS to Enable "

040011 DescSize         =0x28

040010 Desc             ="Minimum %TPS to Enable Power Enrichment"

040100 Address          =0x501

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040210 DecimalPl        =0x1

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x5

040305 Cols             =0x1

040320 XUnits           ="%TPS"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  400, 1200, 2000, 2800, 3600

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xB4

040005 Title            ="PE Knock Retard Added vs LV8 in PE Mode (Wot)"

040011 DescSize         =0x54

040010 Desc             ="Max Knock Retard Degrees when in Power Enrichment Mode  Maximum Knock Retard vs LV8"

040100 Address          =0x1D4

040200 ZEq              =0.175800 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="Degrees"

040325 YUnits           ="LV8"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =00,32,64,96,128,160,192,224,256

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040425 YALDLObj         =0xC

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xB8

040005 Title            ="PE Spark Advance"

040011 DescSize         =0x24

040010 Desc             ="Power Enrichment Mode Spark Advance"

040100 Address          =0x187

040200 ZEq              =0.351567 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x5

040305 Cols             =0x1

040320 XUnits           ="Degrees"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =400,1200,2000,3200,4800

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x3BE6

040005 Title            ="PE Min TPS Volts Enable PE"

040011 DescSize         =0x2D

040010 Desc             ="Minimum TPS Volts to Enable Power Enrichment"

040100 Address          =0x501

040200 ZEq              =X/51,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x5

040305 Cols             =0x1

040320 XUnits           ="TPS Volts"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =Volts

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  400, 1200, 2000, 2800, 3600

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xBC

040005 Title            ="PE Min % TPS to Enable (Hot Coolant)"

040011 DescSize         =0x36

040010 Desc             ="Minimum %TPS to Enable Power Enrichment (Hot Coolant)"

040100 Address          =0x506

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040210 DecimalPl        =0x1

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x5

040305 Cols             =0x1

040320 XUnits           ="%TPS"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  400, 1200, 2000, 2800, 3600

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xDFB

040005 Title            ="PE Min TPS Volts to Enable (Hot Coolant)"

040011 DescSize         =0x3B

040010 Desc             ="Minimum TPS Volts to Enable Power Enrichment (Hot Coolant)"

040100 Address          =0x506

040200 ZEq              =X/51,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x5

040305 Cols             =0x1

040320 XUnits           ="TPS Volts"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  400, 1200, 2000, 2800, 3600

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xB9

040005 Title            ="PE vs Coolant Temp"

040011 DescSize         =0x21

040010 Desc             ="Power Enrichment vs Coolant Temp"

040100 Address          =0x50C

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="% Enrichment"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40,03,08,46,133,176,219,262,306

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x6D37

040005 Title            ="PE vs Coolant Temp Decimal"

040011 DescSize         =0x21

040010 Desc             ="Power Enrichment vs Coolant Temp"

040100 Address          =0x50C

040200 ZEq              =X,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040210 DecimalPl        =0x0

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="% Enrichment"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40,03,08,46,133,176,219,262,306

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xBA

040005 Title            ="PE vs RPM"

040011 DescSize         =0x19

040010 Desc             ="Power Enrichment vs. RPM"

040100 Address          =0x515

040200 ZEq              =0.781225 * X + -100.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040210 DecimalPl        =0x1

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="% Enrichment"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =00,400,800,1200,1600,2000,2400,2800,3200,3600,4000,4400,4800,5200,5600,6000,6400

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x46E1

040005 Title            ="PE vs RPM -50"

040011 DescSize         =0x19

040010 Desc             ="Power Enrichment vs. RPM"

040100 Address          =0x515

040200 ZEq              =.390625* X + -50,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040210 DecimalPl        =0x1

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="% Enrichment"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =00,400,800,1200,1600,2000,2400,2800,3200,3600,4000,4400,4800,5200,5600,6000,6400

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x4C03

040005 Title            ="PE vs RPM 128"

040011 DescSize         =0x19

040010 Desc             ="Power Enrichment vs. RPM"

040100 Address          =0x515

040200 ZEq              =(X-128)/1.28,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040210 DecimalPl        =0x1

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="% Enrichment"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =00,400,800,1200,1600,2000,2400,2800,3200,3600,4000,4400,4800,5200,5600,6000,6400

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x4C4A

040005 Title            ="PE vs RPM Decimal"

040011 DescSize         =0x21

040010 Desc             ="Power Enrichment vs. RPM Decimal"

040100 Address          =0x515

040200 ZEq              =X,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040210 DecimalPl        =0x0

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="% Enrichment"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =00,400,800,1200,1600,2000,2400,2800,3200,3600,4000,4400,4800,5200,5600,6000,6400

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xCA

040005 Title            ="AE Factor vs Change in LV8"

040011 DescSize         =0x6E

040010 Desc             ="Acceleration Enrichment Factor vs Change in LV8                          Columns      Factor - Divide by 10  "

040100 Address          =0x358

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x5

040305 Cols             =0x1

040320 XUnits           ="Factor - Divide by 10 "

040325 YUnits           ="LV8"

040330 ZUnits           =""

040350 XLabels          =

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =    0,   64,  128,  192,  256

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xCB

040005 Title            ="AE Decay Factor vs Coolant Temp"

040011 DescSize         =0x3C

040010 Desc             ="Acceleration Enrichment Decay Factor vs Coolant Temperature"

040100 Address          =0x368

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="% Change - Div. by 10"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  -40,    3,   46,   90,  133,  176,  219,  262,  306

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xCC

040005 Title            ="AE Async Pulses vs Cool Temp."

040011 DescSize         =0x31

040010 Desc             ="Number of AE Async Pulses vs Coolant Temperature"

040100 Address          =0x337

040200 ZEq              =1.000000 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="# pulses"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  -40,    3,   46,   90,  133,  176,  219,  262,  306

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xCD

040005 Title            ="AE % BPW Factor vs Async Pulse"

040011 DescSize         =0x34

040010 Desc             ="Acceleration Enrichment % BPW Factor vs Async Pulse"

040100 Address          =0x32E

040200 ZEq              =0.015625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x8

040305 Cols             =0x1

040320 XUnits           ="Factor"

040325 YUnits           ="Pulse"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  1  ,  2  ,  3  ,  4  ,  5  ,  6  ,  7  ,  8

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xCE

040005 Title            ="LV8 AE Cool Factor vs Cool Temp"

040011 DescSize         =0x42

040010 Desc             ="LV8 Acceleration Enrichment Coolant Factor vs Coolant Temperature"

040100 Address          =0x35E

040200 ZEq              =0.007813 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="Multiplier"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  -40,    3,   46,   90,  133,  176,  219,  262,  306

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x1C80

040005 Title            ="O2 Upper 0 Err Ref Slow O2 R/L "

040011 DescSize         =0x26

040010 Desc             ="Upper Zero Error Ref for Slow O2 R/L "

040100 Address          =0x459

040200 ZEq              =X/226,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040210 DecimalPl        =0x3

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="mv"

040325 YUnits           ="Gr/sec"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =0,8,16,24,32,40,48,56,64

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xDF1

040005 Title            ="O2 Upper 0 Err Ref Slow O2 R/L Dec"

040011 DescSize         =0x25

040010 Desc             ="Upper Zero Error Ref for Slow O2 R/L"

040100 Address          =0x459

040200 ZEq              =X,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040205 OutType          =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="mv"

040325 YUnits           ="Gr/sec"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =0,8,16,24,32,40,48,56,64

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x4591

040005 Title            ="O2 Lower 0 Err Ref Slow O2 R/L "

040011 DescSize         =0x53

040010 Desc             ="Lower Zero Error Reference for Slow O2 Rich/Lean Decimal         4.425000*X+0.0000"

040100 Address          =0x462

040200 ZEq              =X/226,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040210 DecimalPl        =0x3

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="mv"

040325 YUnits           ="Gr/sec"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =0,8,16,24,32,40,48,56,64

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x7465

040005 Title            ="O2 Lower 0 Err Ref Slow O2 R/L Dec"

040011 DescSize         =0x56

040010 Desc             ="Lower Zero Error Reference for Slow O2 Rich/Lean Dec                4.425000*X+0.0000"

040100 Address          =0x462

040200 ZEq              =X,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040205 OutType          =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="mv"

040325 YUnits           ="Gr/sec"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =0,8,16,24,32,40,48,56,64

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x7292

040005 Title            ="O2 C/L R/L Threshold vs  Airflow "

040011 DescSize         =0x56

040010 Desc             ="Closed Loop Rich/Lean Threshold vs. Airflow                   4.425000 * X + 0.000000"

040100 Address          =0x46B

040200 ZEq              =X/226,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040210 DecimalPl        =0x3

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="mvolt"

040325 YUnits           ="gm/sec"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =00,08,16,24,32,40,48,56,64

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xBE

040005 Title            ="O2 C/L R/L Threshold vs  Airflow Dec"

040011 DescSize         =0x59

040010 Desc             ="Closed Loop Rich/Lean Threshold vs. Airflow Dec                  4.425000 * X + 0.000000"

040100 Address          =0x46B

040200 ZEq              =X,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040205 OutType          =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="mvolt"

040325 YUnits           ="gm/sec"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =00,08,16,24,32,40,48,56,64

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xDA

040005 Title            ="O2 Error Reduction Gain Vs Airflow"

040011 DescSize         =0x23

040010 Desc             ="O2 Error Reduction Gain Vs Airflow"

040100 Address          =0x486

040200 ZEq              =0.003906 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040210 DecimalPl        =0x3

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="Multiplier"

040325 YUnits           ="Grams/Sec"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =    0,    8,   16,   24,   32,   40,   48,   56,   64

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xBD

040005 Title            ="O2 Rich/Lean  Offset Vs  Coolant"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x4AD

040200 ZEq              =1.000000 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="Offset"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40,-18,03,25,46,68,90,111,133,154,176,198,219,241

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xBF

000100 Cat0ID           =0x1

040005 Title            ="MAF Table #1"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x5B5

040150 Flags            =0x30

040200 ZEq              =X * s / 256,TH|0|0|0|0|,OB|22|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =23.9100

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="Grams/sec"

040325 YUnits           ="Volts"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =00.00,00.18,00.37,00.55,00.73,00.91,01.10,01.28,01.46

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xC0

000100 Cat0ID           =0x1

040005 Title            ="MAF Table #2"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x5C0

040150 Flags            =0x30

040200 ZEq              =X * s / 256,TH|0|0|0|0|,OB|23|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =47.8100

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="Grams/sec"

040325 YUnits           ="Volts"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =01.46,01.55,01.65,01.74,01.83,01.92,02.01,02.20,02.19

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xC1

000100 Cat0ID           =0x1

040005 Title            ="MAF Table #3"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x5CB

040150 Flags            =0x30

040200 ZEq              =X * s / 256,TH|0|0|0|0|,OB|24|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =80.6800

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="Grams/sec"

040325 YUnits           ="Volts"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =02.19,02.29,02.38,02.47,02.56,02.65,02.74,02.83,02.93

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xC2

000100 Cat0ID           =0x1

040005 Title            ="MAF Table #4"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x5D6

040150 Flags            =0x30

040200 ZEq              =X * s / 256,TH|0|0|0|0|,OB|25|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =130.4900

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="Grams/sec"

040325 YUnits           ="Volts"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =02.93,03.02,03.11,03.20,03.29,03.38,03.47,03.57,03.66

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xC3

000100 Cat0ID           =0x1

040005 Title            ="MAF Table #5"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x5E1

040150 Flags            =0x30

040200 ZEq              =X * s / 256,TH|0|0|0|0|,OB|26|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =203.2000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="Grams/sec"

040325 YUnits           ="Volts"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =03.66,03.75,03.84,03.93,04.02,04.11,04.21,04.30,04.39

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xC4

000100 Cat0ID           =0x1

040005 Title            ="MAF Table #6"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x5EC

040150 Flags            =0x30

040200 ZEq              =X * s / 256,TH|0|0|0|0|,OB|57D3|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="Grams/sec"

040325 YUnits           ="Volts"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =04.39,04.43,04.48,04.52,04.57,04.62,04.66,04.71,04.75,04.80,04.85,04.89,04.94,04.98,05.03,05.07,05.12

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xC5

000100 Cat0ID           =0x1

040005 Title            ="MAF Max Air Flow vs RPM"

040011 DescSize         =0x18

040010 Desc             ="Maximum Air Flow vs RPM"

040100 Address          =0x600

040150 Flags            =0x30

040200 ZEq              =1.000000 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x2

040304 YOutType         =0x2

040210 DecimalPl        =0x0

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="Grs/sec"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =00

040352 XLabelType       =0x2

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =00,400,800,1200,1600,2000,2400,2800,3200,3600,4000,4400,4800,5200,5600,6000,6400

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040425 YALDLObj         =0x4

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x7C8A

040005 Title            ="MAF Default Air Flow vs RPM"

040011 DescSize         =0x32

040010 Desc             ="Default Air Flow Offset Per Percentage TPS VS RPM"

040100 Address          =0x223

040200 ZEq              =X,TH|0|0|0|0|

040203 XOutType         =0x2

040304 YOutType         =0x4

040210 DecimalPl        =0x0

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="Grs/sec"

040325 YUnits           ="rpm"

040330 ZUnits           =""

040350 XLabels          =00

040352 XLabelType       =0x2

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =400,800,1200,1600,2000,2400,3200,4000,4800

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xD8

040005 Title            ="Integrator Delay Vs Airflow"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x474

040200 ZEq              =12.500000 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="mSec"

040325 YUnits           ="Grams/Sec"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =    0,    8,   16,   24,   32,   40,   48,   56,   64

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xD9

040005 Title            ="Integrator Delay Multiplier Vs Error"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x4BB

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xC

040305 Cols             =0x1

040320 XUnits           ="Multiplier"

040325 YUnits           ="Error"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =    0,    8,   16,   24,   32,   40,   48,   56,   64,   72,   80,   88

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xC8

040005 Title            ="Highway Mode Air/Fuel Ratio vs LV8"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x441

040200 ZEq              =0.100000 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x4

040305 Cols             =0x1

040320 XUnits           ="Air/Fuel Ratio"

040325 YUnits           ="LV8"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =   32,   64,   96,  128

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xC6

000100 Cat0ID           =0x3

040005 Title            ="Idle RPM vs Coolant Temperature"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x52E

040150 Flags            =0x30

040200 ZEq              =12.500000 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x2

040230 RangeLow         =650.0000

040240 RangeHigh        =1200.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="RPM"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =-40,-18,03,24,46,68,90,111,133,154,176,198,219,241,262,284,306

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040425 YALDLObj         =0x4

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xD7

040005 Title            ="IAC Warm Park Steps vs Cool Temp"

040011 DescSize         =0x2B

040010 Desc             ="Warm IAC Park Steps vs Coolant Temperature"

040100 Address          =0x588

040200 ZEq              =1.000000 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="steps"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  -40,    3,   46,   90,  133,  176,  219,  262,  306

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xCF

040005 Title            ="Inj Pulse Width Cor vs Bat Volts"

040011 DescSize         =0x33

040010 Desc             ="Injector Pulse Width Correction vs Battery Voltage"

040100 Address          =0x3B5

040200 ZEq              =30.518000 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x11

040305 Cols             =0x1

040320 XUnits           ="usec added"

040325 YUnits           ="VDC"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  0.0,  1.6,  3.2,  4.8,  6.4,  8.0,  9.6, 11.2, 12.8, 14.4, 16.0, 17.6, 19.2, 20.8, 22.4, 24.0, 25.6

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x7B08

040005 Title            ="Low Inj Pulse Width Offset vs Base Pulse Width ms"

040011 DescSize         =0x83

040010 Desc             ="Low Pulse Width Injector Offset vs Base Pulse Width                    Injector Offset for Small Pulse Width's vs Base Pulse Width"

040100 Address          =0x3C6

040200 ZEq              =X*.01526,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040210 DecimalPl        =0x3

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="ms offset"

040325 YUnits           ="IBPW"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =.488,.732,.976,1.220,1.460,1.708,1.950,2.197,2.440,2.685,2.929,3.170,3.410,3.660

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x4EC2

040005 Title            ="Low Inj Pulse Width Offset vs Base Pulse Width us"

040011 DescSize         =0x83

040010 Desc             ="Low Pulse Width Injector Offset vs Base Pulse Width                    Injector Offset for Small Pulse Width's vs Base Pulse Width"

040100 Address          =0x3C6

040200 ZEq              =X*15.26,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xE

040305 Cols             =0x1

040320 XUnits           ="us offset"

040325 YUnits           ="IBPW"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =.488,.732,.976,1.220,1.460,1.708,1.950,2.197,2.440,2.685,2.929,3.170,3.410,3.660

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xD0

040005 Title            ="TCC Unlock Speed vs %TPS (Not In 4th)"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x2C0

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xC

040305 Cols             =0x1

040320 XUnits           ="%TPS"

040325 YUnits           ="MPH"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =   12,   20,   28,   36,   44,   52,   60,   68,   76,   84,   92,  100

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x609F

040005 Title            ="TCC Unlock Speed vs TPS Volts (Not In 4th)"

040011 DescSize         =0x2B

040010 Desc             ="TCC Unlock Speed vs TPS Volts (Not In 4th)"

040100 Address          =0x2C0

040200 ZEq              =X/51,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xC

040305 Cols             =0x1

040320 XUnits           ="%TPS"

040325 YUnits           ="MPH"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =   12,   20,   28,   36,   44,   52,   60,   68,   76,   84,   92,  100

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xD1

040005 Title            ="TCC Lock Speed vs %TPS (Not In 4th)"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x2B4

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xC

040305 Cols             =0x1

040320 XUnits           ="%TPS"

040325 YUnits           ="MPH"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =   12,   20,   28,   36,   44,   52,   60,   68,   76,   84,   92,  100

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x304F

040005 Title            ="TCC Lock Speed vs TPS Volts (Not In 4th)"

040011 DescSize         =0x29

040010 Desc             ="TCC Lock Speed vs TPS Volts (Not In 4th)"

040100 Address          =0x2B4

040200 ZEq              = X/51,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xC

040305 Cols             =0x1

040320 XUnits           ="%TPS"

040325 YUnits           ="MPH"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =   12,   20,   28,   36,   44,   52,   60,   68,   76,   84,   92,  100

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xD2

040005 Title            ="TCC Unlock Speed vs %TPS (In 4th)"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x2DC

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xC

040305 Cols             =0x1

040320 XUnits           ="%TPS"

040325 YUnits           ="MPH"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =   12,   20,   28,   36,   44,   52,   60,   68,   76,   84,   92,  100

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x3EAA

040005 Title            ="TCC Unlock Speed vs TPS Volts (In 4th)"

040011 DescSize         =0x27

040010 Desc             ="TCC Unlock Speed vs TPS Volts (In 4th)"

040100 Address          =0x2DC

040200 ZEq              = X/51 ,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xC

040305 Cols             =0x1

040320 XUnits           ="TPS Volts"

040325 YUnits           ="MPH"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =   12,   20,   28,   36,   44,   52,   60,   68,   76,   84,   92,  100

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xD3

040005 Title            ="TCC Lock Speed vs %TPS (In 4th)"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x2D0

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xC

040305 Cols             =0x1

040320 XUnits           ="%TPS"

040325 YUnits           ="MPH"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =   12,   20,   28,   36,   44,   52,   60,   68,   76,   84,   92,  100

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x20ED

040005 Title            ="TCC Lock Speed vs TPS Volts (In 4th)"

040011 DescSize         =0x25

040010 Desc             ="TCC Lock Speed vs TPS Volts (In 4th)"

040100 Address          =0x2D0

040200 ZEq              =X/51,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xC

040305 Cols             =0x1

040320 XUnits           ="TPS Volts"

040325 YUnits           ="MPH"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =   12,   20,   28,   36,   44,   52,   60,   68,   76,   84,   92,  100

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xDC

040005 Title            ="Shift Light On Table"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x2F3

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x7

040305 Cols             =0x1

040320 XUnits           ="%TPS"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  800, 1600, 2400, 3200, 4000, 4800, 5600

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xDD

040005 Title            ="Shift Light Off Table"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x2FC

040200 ZEq              =0.390625 * X + 0.000000,TH|0|0|0|0|

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x7

040305 Cols             =0x1

040320 XUnits           ="%TPS"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  800, 1600, 2400, 3200, 4000, 4800, 5600

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xDB

040005 Title            ="Shift Light Time On (sec's vs %TPS)"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x304

040200 ZEq              =0.100000 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x5

040305 Cols             =0x1

040320 XUnits           ="Seconds"

040325 YUnits           ="%TPS"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  0.0, 12.5, 25.0, 37.5, 50.0

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xC7

040005 Title            ="EGR Duty Cycle vs LV8 and RPM"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x24B

040200 ZEq              =0.391000 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x2

040304 YOutType         =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x4

040305 Cols             =0x9

040320 XUnits           ="LV8"

040325 YUnits           ="RPM"

040330 ZUnits           =""

040350 XLabels          =32,48,64,80,96,112,128,144,160

040352 XLabelType       =0x2

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =800,1200,1600,2000

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040400 XALDLObj         =0xC

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xF76

040005 Title            ="CCP DC Limit vs AirFlow"

040011 DescSize         =0x18

040010 Desc             ="CCP DC LIMIT vs AIRFLOW"

040100 Address          =0x287

040200 ZEq              =0.391000 * X + 0.000000,TH|0|0|0|0|

040203 XOutType         =0x2

040304 YOutType         =0x2

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x9

040305 Cols             =0x1

040320 XUnits           ="DC"

040325 YUnits           ="Grs/sec"

040330 ZUnits           =""

040350 XLabels          =32

040352 XLabelType       =0x2

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =00,04,08,12,16,20,24,28,32

040362 YLabelType       =0x2

040364 YEq              =X,TH|0|0|0|0|

040400 XALDLObj         =0xC

040425 YALDLObj         =0x9

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xB5

040005 Title            ="Fan DC vs. Coolant Temp F                  (0.390625 * X )"

040100 Address          =0x311

040200 ZEq              =0.390625 * X ,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0x7

040305 Cols             =0x1

040320 XUnits           ="Fan % Duty Cycle"

040325 YUnits           ="Deg F"

040330 ZUnits           =""

040350 XLabels          =%

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =  176,  198,  219,  241,  262,  284,  306

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0x12D3

040005 Title            ="Fuel limit fix"

040011 DescSize         =0x2C

040010 Desc             ="Hex changes to fix a fuel limit in OEM bins"

040100 Address          =0x1CDB

040200 ZEq              =X,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040205 OutType          =0x3

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xF

040305 Cols             =0x1

040320 XUnits           ="Hex"

040325 YUnits           ="Location"

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =1CDB,1CDC,1CDD,1CDE,1CDF,1CE0,1CE1,1CE2,1CE3,1CE4,1CE5,1CE6,1CE7,1CE8,1CE9

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%TABLE%%

000002 UniqueID         =0xE92

040005 Title            ="Copy of New Table 1"

040011 DescSize         =0x1

040010 Desc             =""

040100 Address          =0x1CDB

040200 ZEq              =X,TH|0|0|0|0|

040203 XOutType         =0x4

040304 YOutType         =0x4

040205 OutType          =0x3

040230 RangeLow         =0.0000

040240 RangeHigh        =255.0000

040300 Rows             =0xF

040305 Cols             =0x1

040320 XUnits           =""

040325 YUnits           =""

040330 ZUnits           =""

040350 XLabels          =(null)

040352 XLabelType       =0x4

040354 XEq              =X,TH|0|0|0|0|

040360 YLabels          =1CDB

040362 YLabelType       =0x4

040364 YEq              =X,TH|0|0|0|0|

040660 XAxisMin         =1000.000000

040670 XAxisMax         =1000.000000

040760 YAxisMin         =1000.000000

040770 YAxisMax         =1000.000000

%%END%%

%%FLAG%%

000002 UniqueID         =0x1DA2

030005 Title            ="O2 Diagnostic Enable (Error Code 13)"

030011 DescSize         =0x6D

030010 Desc             ="Enable Error Code 13 Diagnostic                                             (Unset) Disables ERR Recognition"

030100 Address          =0x1DD

030200 BitNumber        =0x6

%%END%%

%%FLAG%%

000002 UniqueID         =0x537

030005 Title            ="MAF Diagnostic Enable (Error Code 34) Sensor Low"

030011 DescSize         =0x6D

030010 Desc             ="Enable Error Code 34 Diagnostic                                             (Unset) Disables ERR Recognition"

030100 Address          =0x1DE

030200 BitNumber        =0x1

%%END%%

%%FLAG%%

000002 UniqueID         =0x5848

030005 Title            ="MAF Diagnostic Enable (Error Code 33) Sensor High"

030011 DescSize         =0x6D

030010 Desc             ="Enable Error Code 33 Diagnostic                                             (Unset) Disables ERR Recognition"

030100 Address          =0x1DE

030200 BitNumber        =0x2

%%END%%

%%FLAG%%

000002 UniqueID         =0x4613

030005 Title            ="EGR Diagnostic Enable (Error Code 32) "

030011 DescSize         =0x6C

030010 Desc             ="Enable Error Code 32 Diagnostic                                            (Unset) Disables ERR Recognition"

030100 Address          =0x1DE

030200 BitNumber        =0x3

%%END%%

%%FLAG%%

000002 UniqueID         =0x5AD

030005 Title            ="VSS Diagnostic Enable (Error Code 24) "

030011 DescSize         =0x6C

030010 Desc             ="Enable Error Code 24 Diagnostic                                            (Unset) Disables ERR Recognition"

030100 Address          =0x1DE

030200 BitNumber        =0x7

%%END%%

%%FLAG%%

000002 UniqueID         =0x39E5

030005 Title            ="ESC Diagnostic Enable (Error Code 43)"

030011 DescSize         =0x6A

030010 Desc             ="Enable Error Code 43 Diagnostic                                          (Unset) Disables ERR Recognition"

030100 Address          =0x1DF

030200 BitNumber        =0x4

%%END%%